# Flight Planning Checklist

- Select Date and Launch Time

  - Date: 4 / 12 / 2021

  - Time: 10:30 AM CST

- Weather Conditions (https://www.wunderground.com/forecast/us/al/auburn/32.62,-85.49)

  - Cloud cover < 50% : 6 %

  - Rain < 30% : 1 %

  - Jet Stream < 100 knots: 70 knots (https://weatherstreet.com/models/gfs-jetstream-wind-forecast.php)

  - Ground Speed Winds < 12 mph: 2mph

- Calculate Balloon Dynamics

  - Payload mass: 950 g

  - Balloon mass: 600 g

  - Positive lift: 875 g  (http://tools.highaltitudescience.com/)

  - Total lift: 1825 g

  - Required helium: 87.18 cu-f (http://tools.highaltitudescience.com/)

  - Ascent Rate: 5.04 m/s (http://tools.highaltitudescience.com/)

  - Descent Rate: 7.1 m/s (https://www.highaltitudescience.com/products/0-9-m-parachute)

  - Burst Altitude: 27450 m

- Launch and Landing Location

  - Launch Address (https://www.freemaptools.com/elevation-finder.htm)

Uniontown Ball Park, Uniontown, AL 36786

Lat: 32.4519, Lon: -87.5074 Elev: 87.0m

    - Outside of restricted aerospace (https://skyvector.com/)

  - Predict landing zone (https://predict.habhub.org/)

Lat: 32.2346, Lon: -86.2678 Elev: 77m

- Flight Dynamics

  - Flight Time: 2 hrs, 2 min Landing Time: 1:32 PM

  - Total Distance: 73 mi

  - Driving distance (Launch to Recovery): 83 mi, Driving time: 1 hrs, 29 min

  - Driving distance (Round Trip): 283 mi, Driving time: 5 hrs, 0 min

- File NOTEM: (http://blogs.und.edu/jdosas/wp-content/uploads/sites/108/2017/12/Instructions-Filing-a-NOTAM.pdf)

  - Call (877) 487-6867

  - Say “Hi, I would like to file a NOTAM for an Unmanned Balloon Flight”

  - Launch Location in Lat Lon with Degrees,Min,Second: (https://www.fcc.gov/media/radio/dms-decimal)

  - Lat: 32° 27' 6.84" Lon: -87° 30' 26.64"

  - Distance to nearest airport: 6.9 nmi

  - Atmosphere Level: Surface to Unlimited (appr 90,000ft)

  - Direction of flight: South-East

  - Time Effective:

    - 10:15am to 1:00pm CST or 1515 to 1800 Zulu on 4/12

  - My Initials: MTC (Mike Tango Charlie)

  - NOTEM Number: ZTL04/217 by GM

  - Filed Time: 11:48am on 4/10

# Departure Checklist

- Packed the following:

    - Balloon

    - Payload

    - Helium tank and inflator

    - Batteries

    - Toolbox

    - Wireless router

    - APRS receiver

- No USB mouse dongle receiver in Pi

# Pre-Flight Checklist

- Payload Preparation

- Identify tall obstacles: light posts, power lines, trees

- Identify wind direction (if applicable)

- Lay down tarp/blanket in location to maximize balloon travel distance to tall objects

- Setup router

- Remove lens cap

- Plug-in battery

- Connect via ssh

- Start ROS and verify sensors are working

- Calibrate IMUs

- Verify Spot3 is tracking

- Verify APRS signal is being transmitted via Direwolf

- Use duct tape to seal box

- Start recording ROS bag

- Balloon inflation (during payload preparation)

- Safety rope attached to payload and balloon from tank

- Balloon inflated to provide ______ g lifting force and tied off

- Take pictures of balloon inflation process

- Inspect train rope and knots

- Launch balloon

- Disengage safety rope from balloon

- Disengage safety rope from payload

- Record launch time ____:_____ _____

- Take video of launch process

# Post-Flight Checklist

- Upon Discovery of Payload

- Take picture of payload before touching and moving it

- Cut open box, disconnect battery

- Record landing site: Lat: ___________ Lon: ___________ Elev: __________

- Record recovery time ____:_____ _____